EC42:00-6

One Hour Program Format

This clock used by the following networks:

| Description (SMPTE drop frame time code is mandatory) |  | Timecode In (29.97) | Timecode In (25) | Duration | Timecode Out (29.97) | Timecode Out (25) |
| --- | --- | --- | --- | --- | --- | --- |
| Bars & Tone |  | 00;59;40;00 | 09:59:40:00 | 00:04:00 | 00;59;44;00 | 09:59:44:00 |
| Black | Black | 00;59;44;00 | 00;59;44;00 | 00:01:00 | 00;59;45;00 | 09:59:45:00 |
| Slate | Slate(s) main + supplemental (woos, other) :05 each | 00;59;45;00 | 09:59:45:00 | 00:10:00 | 00;59;55;00 | 09:59:55:00 |
| Countdown | Countdown from :05 to :03 (with audible tone at each 1 second interval) | 00;59;55;00 | 09:59:55:00 | 00:03:00 | 00;59;57;02 | 09:59:57:02 |
| Black | Black | 00;59;57;02 | 09:59:57:02 | 00:00:06 | 00;59;57;06 | 09:59:57:06 |
| White | Sync Indicator (two frames of white video and 1 khz tone) | 00;59;57;06 | 09:59:57:06 | 00:00:02 | 00;59;57;08 | 09:59:57:08 |
| Black | Black | 00;59;57;08 | 09:59:57:08 | 00:02:20 (NTSC) 00:02:15 (PAL) | 01:00;00;00 | 10:00:00:00 |
| ## Content Act #1 | Act #1 (Should be longest act. Not to exceed 15 min.) | 01;00;00;00 | 10:00:00:00 |  |  |  |
| BREAK #1 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #2 | Act #2 (Should be the second longest act, or else on the longer side of total content time) |  |  |  |  |  |
| BREAK #2 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #3 | Act #3 (Act length minimum 3 min, with preference to be longer) |  |  |  |  |  |
| BREAK #3 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #4 | Act #4 (Act length minimum 3 min, with preference to be longer) |  |  |  |  |  |
| BREAK #4 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #5 | Act #5 (Act length minimum 3 min, with preference to be longer) |  |  |  |  |  |
| BREAK #5 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #6 | Act #6 (Act length minimum 3 min) |  |  |  | 01;42;25;00 | 10:42:25:00 |
| ## Credits | *See Credits Note (on page 2) | 01;41;55;00 | 10:41:55:00 | 00:30:00 | 01;42;25;00 | 10:42:25:00 |

Continued next page

EC42:00-6

One Hour Program Format

PROGRAM FORMAT SPECIFICATIONS FOR NTSC/PAL:

- First program video and/or audio must hit at                                                                       timecode 01;00;00;00 (29.97) / 10:00:00:00 (25).

- Total Content Time	including embedded credits (cannot exceed)			42:00

(Content Time Includes: disclaimers, program tease/open, graphic titles, Acts 1-6, bumps in/out,

embedded credits and any other elements, (promotional, ad sales, etc.) attached to the program.)

- Breaks (:05 x 5 breaks)									00:25

- TOTAL RUNNING TIME (TRT)								42:25

*Credits Note - CREDITS must be embedded over the video at the end of the last segment, preferably the last :30 seconds. Embedded credits can start earlier if there is a need to have the final seconds of video clean at the end of the program. The credits MUST be :30 seconds long. Credits must “page” on and off. See the Producer’s Portal and your Discovery Production Team for details.

Editing Guidelines:

- Segments do not have to begin and end at :00 frames, except the beginning of segment 1 and the end of segment 6.

- Act #1 should be the longest act in the program. Not to exceed 15 minutes, unless given sign off by EP.

- Act #2 should be the second longest act, or else on the longer side of total content time.

- Act #6 is preferred to be the shortest act if necessary (versus acts 3 - 5). Total content time cannot dip below 3 minutes, unless given approval by EP.

- See your Discovery Production Team for details if there are any questions regarding segment length, bumps/teases, embedded credits, etc.

Snap In Guidelines:

- Total content time must become 48 minutes of seamless material, not including bumps and teases (if present). Therefore, snap in content delivered needs reach at least 6 minutes total content time.

- Approx. 3 - 5 snap ins should be delivered against the appropriate “Snap In” media deliverable (separately from the program master file). This content can be a stand-alone scene, or additional content that extends a scene inside the episode.

Continued next page

*Special Note – The information on this page applies to programs delivering to Discovery in 1080p 23.98. After Discovery converts the program to 1080i 59.94 with 29.97 drop frame time code the program will match the EC4200-6 clock.

PROGRAM FORMAT SPECIFICATIONS FOR 1080p 23.98:

- First program video and/or audio must hit at time code 01:00:00:00.

- Total Content Time	including embedded credits (cannot exceed)			41:57:13

(Content Time Includes: disclaimers, program tease/open, graphic titles, Acts 1-6, bumps in/out,

embedded credits and any other elements, (promotional, ad sales, etc.) attached to the program.)

- Breaks (:05 x 5 breaks)									00:25:00

- TOTAL RUNNING TIME (TRT)								42:22:11