EC87:30-9

Two Hour Program Format

This clock used by the following networks: MOTORTREND

| Description (SMPTE drop frame time code is mandatory) |  | Timecode In hh;mm;ss;ff (29.97) | Timecode In hh:mm:ss:ff (25) | Duration Mm;ss;ff | Timecode Out Hh;mm;ss;ff (29.97) | Timecode Out Hh:mm:ss:ff (25) |
| --- | --- | --- | --- | --- | --- | --- |
| Bars & Tone |  | 00;59;40;00 | 09:59:40:00 | 00:04:00 | 00;59;44;00 | 09:59:44:00 |
| Black | Black | 00;59;44;00 | 00;59;44;00 | 00:01:00 | 00;59;45;00 | 09:59:45:00 |
| Slate | Slate(s) main + supplemental (woos, other) :05 each | 00;59;45;00 | 09:59:45:00 | 00:10:00 | 00;59;55;00 | 09:59:55:00 |
| Countdown | Countdown from :05 to :03 (with audible tone at each 1 second interval) | 00;59;55;00 | 09:59:55:00 | 00:03:00 | 00;59;57;02 | 09:59:57:02 |
| Black | Black | 00;59;57;02 | 09:59:57:02 | 00:00:06 | 00;59;57;06 | 09:59:57:06 |
| White | Sync Indicator (two frames of white video and 1 khz tone) | 00;59;57;06 | 09:59:57:06 | 00:00:02 | 00;59;57;08 | 09:59:57:08 |
| Black | Black | 00;59;57;08 | 09:59:57:08 | 00:02:20 (NTSC) 00:02:15 (PAL) | 01:00;00;00 | 10:00:00:00 |
| ## Content Act #1 | Act #1 (Should be longest act) | 01;00;00;00 | 10:00:00:00 |  |  |  |
| BREAK #1 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #2 | Act #2 |  |  |  |  |  |
| BREAK #2 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #3 | Act #3 |  |  |  |  |  |
| BREAK #3 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #4 | Act #4 |  |  |  |  |  |
| BREAK #4 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #5 | Act #5 |  |  |  |  |  |
| BREAK #5 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #6 | Act #6 |  |  |  |  |  |
| BREAK #6 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #7 | Act #7 |  |  |  |  |  |
| BREAK #7 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #8 | Act #8 |  |  |  |  |  |
| BREAK #8 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #9 | Act #9 |  |  |  | 02:28:10:00 | 11:28:10:00 |
| ## Credits | *See Credits Note (on page 2) | 02:27:40:00 | 11:27:40:00 |  | 02:28:10:00 | 11:28:10:00 |

**Continued from page 1

EC87:30-9

Two Hour Program Format

PROGRAM FORMAT SPECIFICATIONS FOR NTSC/PAL:

- First program video and/or audio must hit at                                                                       timecode 01;00;00;00 (29.97) / 10:00:00:00 (25).

- Total Content Time	including embedded credits (cannot exceed)			87:30

(Content Time Includes: disclaimers, program tease/open, graphic titles, Acts 1-9, bumps in/out,

embedded credits and any other elements, (promotional, ad sales, etc.) attached to the program.)

- Breaks (:05 x 8 breaks)									00:40

- TOTAL RUNNING TIME (TRT)								88:10

*Credits Note - CREDITS must be embedded over the video at the end of the last segment, preferably the last :30 seconds. Embedded credits can start earlier if there is a need to have the final seconds of video clean at the end of the program. The credits MUST be :30 seconds long. Credits must “page” on and off. See your Discovery Production Team for details.

Editing Guidelines:

- Segments do not have to begin and end at :00 frames, except the beginning of segment 1 and the end of segment 9.

- Act #1 should be the longest act in the program.

- Acts #2-#9 should be no shorter than 3 minute of content time.

- See your Discovery Production Team for details if there are any questions regarding segment length, bumps/teases, embedded credits, etc.

*Special Note – The information below applies to programs delivering to Discovery in 1080p 23.98. After Discovery converts the program to 1080i 59.94 with 29.97 drop frame time code the program will match the EC8730-9 clock.

PROGRAM FORMAT SPECIFICATIONS FOR 1080p 23.98:

- First program video and/or audio must hit at time code 01:00:00:00.

- Total Content Time	including embedded credits (cannot exceed)			87:24:18

(Content Time Includes: disclaimers, program tease/open, graphic titles, Acts 1-9, bumps in/out,

embedded credits and any other elements, (promotional, ad sales, etc.) attached to the program.)

- Breaks (:05 x 8 breaks)									00:40:00

- TOTAL RUNNING TIME (TRT)								88:04:18