AM EC12600-16  	Three Hour Program Format

16 Act - 15 Break

This clock used by the following networks:

| Description (SMPTE drop frame time code is mandatory) |  | Timecode In hh;mm;ss;ff (NTSC) | Timecode In hh:mm:ss:ff (PAL) | Duration Mm;ss;ff | Timecode Out Hh;mm;ss;ff (NTSC) | Timecode Out Hh:mm:ss:ff (PAL) |
| --- | --- | --- | --- | --- | --- | --- |
| Bars & Tone |  | 00;58;30;00 | 09:58:30:00 | 01:00:00 | 00;59;30;00 | 09:59:30:00 |
| Black | Black | 00;59;30;00 | 09:59:30:00 | 00:05:00 | 00;59;35;00 | 09:59:35:00 |
| Slate | :15 slate | 00;59;35;00 | 09:59:35:00 | 00:15:00 | 00;59;50;00 | 09:59:50:00 |
| Countdown | Countdown from :10 to :03 (with audible tone at each 1 second interval) | 00;59;50;00 | 09:59:50:00 | 00:07:00 | 00;59;57;00 | 10:59:57:00 |
| Black | Black | 00;59;57;00 | 10:59:57:00 | 00:00:06 | 00;59;57;06 | 09:59:57:06 |
| White | Sync Indicator (two frames of white video and 1 khz tone) | 00;59;57;06 | 09:59:57:06 | 00:00:02 | 00;59;57;08 | 09:59:57:08 |
| Black | Black | 00;59;57;08 | 09:59:57:08 | 00:02:22 (NTSC) 00:02:17 (PAL) | 01;00;00;00 | 10:00:00:00 |
| ## Content Act #1 | Program Intro/Tease | 01;00;00;00 | 10:00:00:00 |  |  |  |
|  | Act #1 (should be between 12:00-15:00) |  |  |  |  |  |
| BREAK #1 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #2 | Act #2 |  |  |  |  |  |
| BREAK #2 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #3 | Act #3 |  |  |  |  |  |
| BREAK #3 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #4 | Act #4 |  |  |  |  |  |
| BREAK #4 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #5 | Act #5 |  |  |  | Must end before 01;44;35;00 | Must end before 10:44:35:00 |
| BREAK #5 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #6 | Act #6 |  |  |  |  |  |
| BREAK #6 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #7 | Act #7 |  |  |  |  |  |
| BREAK #7 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #8 | Act #8 |  |  |  |  |  |
| BREAK #8 | Black |  |  | 00:05:00 |  |  |

Continued on next page

AM EC12600-16  	Three Hour Program Format

16 Act - 15 Break

| ## Content Act #9 | Act #9 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BREAK #9 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #10 | Act #10 |  |  |  |  |  |
| BREAK #10 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #11 | Act #11 |  |  |  | Must end before 02;26;20;00 | Must end before 11:26:20:00 |
| BREAK #11 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #12 | Act #12 |  |  |  |  |  |
| BREAK #12 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #13 | Act #13 |  |  |  |  |  |
| BREAK #13 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #14 | Act #14 |  |  |  |  |  |
| BREAK #14 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #15 | Act #15 |  |  |  |  |  |
| BREAK #15 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #16 | Act #16 |  |  |  | 03;07;15;00 | 12:07:15:00 |
| ## Credits | *See Special Note (on page 3) | 03;06;45;00 | 12:06:45:00 | 00:30:00 | 03;07;15;00 | 12:07:15:00 |

Continued on next page

AM EC12600-16  	Three Hour Program Format

16 Act - 15 Break

PROGRAM FORMAT SPECIFICATIONS:

- First program video and/or audio must hit at                                                                       timecode 01;00;00;00 (NTSC) / 10:00:00:00 (PAL).

- Total Content Time	including embedded credits (cannot exceed)			02:06:00

(Content Time Includes: disclaimers, program tease/open, graphic titles, Acts 1-10, bumps in/out,

embedded credits and any other elements, (promotional, adsales, etc.) attached to the program.)

- Breaks (:05 x 15 breaks)									01:15

- TOTAL RUNNING TIME (TRT)								02:07:15

Total running time must adhere to the above specs to the frame.  Failure to do so will result in Technical Evaluation (QC) failure and potential costs to the producer.

*Special Note - CREDITS must be embedded over the video at the end of the last segment, preferably the last :20 seconds. Embedded credits can start earlier if there is a need to have the final seconds of video clean at the end of the program. The credits MUST be :20 seconds long.  Credits must "page" on and off.  See your DCL Production Team for details.  Information can also be found on the PMD website, under Credit Guidelines.

Editing Guidelines:

- Segment 1 begins at 01;00;00;00 (NTSC)/10:00:00:00 (PAL). Segment 16 ends at 03;07;15;00 (NTSC)/ 12:07:15:00 (PAL). Credits must be embedded over content and cannot run beyond 03;07;15;00 (NTSC)/ 12:07:15:00 (PAL).

- Segment 5 must end before 01;44;35;00(NTSC), 10:44:35:00(PAL).

- Segment 11 must end before 02;26;20;00 (NTSC), 11:26:20:00 (PAL).

- Segments do not have to begin and end at ;00 frames, except the beginning of segment 1 and the end of segment 10.

- See your DCL Production Team for details if there are any questions regarding segment length, bumps/teases, embedded credits, etc.

- Snap-in segments ("Additional Program Content"), may also be contractually required; refer to your Program Materials exhibit.