EC21:00-4	Half Hour Program Format

4 Act – 3 Break

(4 Act / 3 Break Version)

This clock used by the following networks:

| Description (SMPTE drop frame time code is mandatory) |  | Timecode In hh;mm;ss;ff (NTSC) | Timecode In hh:mm:ss:ff (PAL) | Duration Mm;ss;ff | Timecode Out Hh;mm;ss;ff (NTSC) | Timecode Out Hh:mm:ss:ff (PAL) |
| --- | --- | --- | --- | --- | --- | --- |
| Bars & Tone |  | 00;58;30;00 | 09:58:30:00 | 01:00:00 | 00;59;30;00 | 09:59:30:00 |
| Black | Black | 00;59;30;00 | 09:59:30:00 | 00:05:00 | 00;59;35;00 | 09:59:35:00 |
| Slate | :15 slate | 00;59;35;00 | 09:59:35:00 | 00:15:00 | 00;59;50;00 | 09:59:50:00 |
| Countdown | Countdown from :10 to :03 (with audible tone at each 1 second interval) | 00;59;50;00 | 09:59:50:00 | 00:07:00 | 00;59;57;00 | 09:59:57:00 |
| Black | Black | 00;59;57;00 | 09:59:57:00 | 00:00:06 | 00;59;57;06 | 09:59:57:06 |
| White | Sync Indicator (two frames of white video and 1 khz tone) | 00;59;57;06 | 09:59:57:06 | 00:00:02 | 00;59;57;08 | 09:59:57:08 |
| Black | Black | 00;59;57;08 | 09:59:57:08 | 00:02:22 (NTSC) 00:02:17 (PAL) | 01;00;00;00 | 10:00:00:00 |
| ## Content Act #1 | Program Intro/Tease | 01;00;00;00 | 10:00:00:00 |  |  |  |
|  | Act #1 |  |  |  |  |  |
| BREAK #1 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #2 | Act #2 |  |  |  |  |  |
| BREAK #2 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #3 | Act #3 |  |  |  |  |  |
| BREAK #3 | Black |  |  | 00:05:00 |  |  |
| ## Content Act #4 | Act #4 |  |  |  | 01;21;15;00 | 10:21:15:00 |
| ## Credits | *See Special Note (on page 2) | 01;20;45;00 | 10:20:45:00 | 00:30:00 | 01;21;15;00 | 10:21:15:00 |

Continued on next page

EC21:00-4	Half Hour Program Format

4 Act – 3 Break

(4 Act / 3 Break Version)

PROGRAM FORMAT SPECIFICATIONS NTSC/PAL:

- First program video and/or audio must hit at                                                                       timecode 01;00;00;00 (NTSC) / 10:00:00:00 (PAL).

- Total Content Time	including embedded credits (cannot exceed)			21:00

(Content Time Includes: disclaimers, program tease/open, graphic titles, Acts 1-4, bumps in/out,

embedded credits and any other elements, (promotional, adsales, etc.) attached to the program.)

- Breaks (:05 x 3 breaks)									00:15

- TOTAL RUNNING TIME (TRT)								21:15

Total running time must adhere to the above specs to the frame.  Failure to do so will result in Technical Evaluation (QC) failure and potential costs to the producer.

*Special Note - CREDITS must be embedded over the video at the end of the last segment, preferably the last :30 seconds. Embedded credits can start earlier if there is a need to have the final seconds of video clean at the end of the program. The credits MUST be :30 seconds long.  Credits are limited to 50 names including 4 reserved for internal DCI usage. Credits must “page” on and off.  See your DCL Production Team for details.

Editing Guidelines:

- Segment 1 begins at 01;00;00;00 (NTSC)/10:00:00:00 (PAL).

Segment 4 ends at 01;21;15;00 (NTSC)/ 10:21:15:00(PAL).

Credits must be embedded over content and cannot run beyond 01;21;15;00 (NTSC)/ 10:21:15:00(PAL).

- Segments do not have to begin and end at ;00 frames, except the beginning of segment 1 and the end of segment 4.

- See your DCL Production Team for details if there are any questions regarding segment length, bump/teases, embedded credits, etc.

- Snap-in segments ("Additional Program Content"), may also be contractually required; refer to your Program Materials exhibit.

EC21:00-4	Half Hour Program Format

4 Act – 3 Break

*Special Note – The information on this page applies to programs delivering to Discovery in 1080p 23.98. After Discovery converts the program to 1080i 59.94 with 29.97 drop frame time code the program will match the EC21:00-4 clock.

PROGRAM FORMAT SPECIFICATIONS FOR 1080p 23.98:

- First program video and/or audio must hit at time code 01:00:00:00.

- Total Content Time	including embedded credits (cannot exceed)			20:88:19

(Content Time Includes: disclaimers, program tease/open, graphic titles, Acts 1-4, bumps in/out,

embedded credits and any other elements, (promotional, ad sales, etc.) attached to the program.)

- Breaks (:05 x 3 breaks)									00:15:00

- TOTAL RUNNING TIME (TRT)								21:13:19

*Special Note - CREDITS must be embedded over the video at the end of the last segment, preferably the last :30 seconds. Embedded credits can start earlier if there is a need to have the final seconds of video clean at the end of the program. The credits MUST be :30 seconds long.  Credits are limited to 50 names including 4 reserved for internal DCI usage. Credits must “page” on and off.  See your DCI Production Team for details.

Editing Guidelines:

- Segment 1 begins at 	01:00:00:00

- Segment 4 ends at 	01:21:13:19.

Credits must be embedded over content and cannot run beyond  01:21:13:18.

- See your DCL Production Team for details if there are any questions regarding segment length, bumps/teases, embedded credits, etc.

- Please refer to your Program Materials exhibit for information regarding Additional Program Content (“Snap-in segments").